MRO-AR2 – Case Study: Assistive Robotics for
Improvement of Life Quality
One‑Page Proposal

| Team | Group 4 |
| --- | --- |
| Members | Purama Manoj Kumar Naveen Naidu |
| Project Title | Simulation of Sensor Fusion for Smart Wheelchair in Adverse Weather Conditions |
| GitHub Repository URL | https://github.com/manojpurama7/Simulation-of-Sensor-Fusion-For-Smart-Wheel-Chair-in-Adverse-Weather-Conditions.git |
| Contact Email | manoj.purama@stud.th-deg.de |
| Industrial Application (target domain/use‑case) | Assistive mobility, specifically a weather-robust LiDAR–radar obstacle avoidance module, with simulation-based sensor evaluation for outdoor use in fog, rain and snow. |
| Keywords (3–6) | sensor fusion; LiDAR–radar, ROS2 Gazebo, Obstacle avoidance |
| Submission Date (YYYY‑MM‑DD) | 2026-10-21 |
| Gant Chart | (Oct) Week 43; 44; (Nov) 45; 46; 47; 48; (Dec) 49; 50; 51; 52; 53; (Jan) 01; 02; 03. Literature map & method outline. Methodology 1. Methodology 2. Demo; experiments & analysis. Final presentation & report. |

| 1) Problem Statement & Measurable Outcomes (3–4 sentences) | - LiDAR and camera perception are common basis for autonomous navigation in smart wheelchairs. Fog, rain and snow degrade LiDAR through light scattering and occlusion, and fusion methods that work in good conditions fail when sensor streams are distorted asymmetrically in adverse weather. Radar is less affected by adverse weather but has lower angular resolution than LiDAR, so the two sensors are complementary. We will therefore simulate LiDAR–radar fusion for wheelchair obstacle avoidance under fog, rain and snow and compare it with a LiDAR-only baseline. - Measurable Outcomes: - obstacle-detection accuracy, precision, recall |
| --- | --- |
| 2) Motivation & Assistive Relevance (2–3 sentences) | Powered-wheelchair users depend on reliable obstacle avoidance, and smart-wheelchair research aims to increase their functional mobility. Adverse-weather cases are rare in available datasets, so simulation is a practical, low-cost and risk-free way to study them before any hardware is built; weather-robust LiDAR–4D-radar fusion is also a current research topic. |
| 3) Related Work Snapshot (2–3 key references) | Qian et al. (CVPR 2021): MVDNet fuses LiDAR and radar for vehicle detection in fog (procedurally generated on Oxford Radar RobotCar) and improves AP, especially in adverse weather. Huang et al. (AAAI 2025): L4DR fuses LiDAR and 4D radar and raises 3D mAP by up to 20.0% over LiDAR-only on VoD with simulated fog. Gap: These works target road-vehicle 3D detection accuracy on driving datasets, not closed-loop wheelchair obstacle avoidance. Our initial search found no closed-loop wheelchair LiDAR–radar study. |
| 4) Method & Feasibility (≤6 sentences) | Environment: A model with static and moving obstacles in ROS 2 and Gazebo, navigated with Nav2. Weather: a ROS 2 node degrades LiDAR scans using physically based fog and snow models at several severity levels |
| 5) Milestones & Timeline (short table/list) | P1 Oct 21 — Proposal P2 Nov 4 — Literature map & method outline; baseline code/sim (submit short Bibliography report 15-20 pages) P3 Nov 18 — Methodology P4 Dec 2 — Methodology P5 Dec 16 — Demo; experiments & analysis P6 Jan 13 — Final presentation & report |
| 6) Risks & Ethics (1–2 sentences) | Main risks are the sim-to-real gap (the fog and snow models, were developed and evaluated on automotive LiDAR data) and the effort of building a credible radar model; we mitigate this by checking against real adverse-weather data and keeping learned fusion optional. The study is simulation-only with no human subjects. |

Phase 1 rubric (15%): Team & GitHub (2%), On‑time (2%), Topic & Proposal (5%) — Industrial Application, Problem+Outcomes, Feasibility+Timeline; Presentation (6%).